Wind Turbine Trial Sheet

A. Power in the Wind:

| 1- Air Density | 2- Turbine Swept Area (m2) | 3- Wind Speed (m/s) | POWER IN THE WIND |
| --- | --- | --- | --- |

B. Turbine Output:

| 1- Trial # | 2-Generator | # Blades | Blade Type | Blade Pitch | Resistor Value | Voltage | Current Amps (note method) | POWER OUTPUT | Turbine efficiency | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Blade Types - Sketch & description:

POWER IN THE WIND:

BELOW: Multimeter set-up to test both voltage & current with a resistor as a load:

Calculate Power available in the wind:   P = ½  ( r2)V3

1- Air Density: Based on estimates of 70 degrees, 50% humidity and 500 ft elevation, the air density () calculator at http://www.denysschen.com/catalogue/density.aspx  gives us an air density reading of about 1.17 kg/m3.

2 – Turbine Swept area: Calculate the area of the circle. The radius is the distance from the center of the hub to the tip of the blade ( r2)

3- Wind speed – take an average of 3 readings from a fan in different places in the air column where you will place your turbine.

Reading 1 __________  Reading 2  ______________  Reading 3 ____________

TURBINE TRIALS - Tips:

*Trial #: Start from 1 again if you change any variable. Change only one variable (blade type, #, or pitch) at a time.

*Avoid pushing too hard on hub since the motor will end up flush with the white piping and blades will scrape.

*When removing black hub or red rotor (off a mini turbine), pry it off very gently with a screwdriver; be careful not to twist.

*Generators: ie – KidWind DC generator; Other DC generator; Wound generator; etc.

*Efficiency calculation: Take the POWER OUTPUT of your turbine and divide it by the POWER IN THE WIND. Multiply by 100 to get % efficiency.

*Be sure dowels are deep in sockets of black test hubs and tighten well.

*Be sure no one, including yourself, is in the plane of rotation of the rotor when it is moving!

REFLECTIONS: Ask yourself questions about the quality of these aspects of turbine design & construction:

Level of Innovation

Quality of construction

Thought put into design

Knowledge of you and your team

Suitability of materials

From KidWind Learn Wind